SECTION 08380

DOUBLE ACTING IMPACT TRAFFIC DOORS

# Part 1 GENERAL

## SUMMARY

- This section includes double acting impact traffic doors.

- Related sections: Include appropriate section below indicating proper frame/jamb details. Recommended jamb is ¼” thick and 4” wide minimum steel frame.

## SUBMITTALS

- Product data:  Submit manufacturer’s data indicating products provided.

- Shop drawings:  Submit indicating locations of required block outs, frame fabrication requirements, sizes, elevations, and other related items.

## DELIVERY, STORAGE AND HANDLING

A.	Packing and shipping:	Deliver work packaged for protection.

B.	Acceptance at site:	 Inspect work upon delivery for damage; rejecting

any damaged items.

C.	Storage and protection:	   Store materials under cover, 4” off floor, minimum, raised platforms, in vertical position with minimum ¼” space between doors.  Avoid use of non vented plastic or canvas shelters creating humidity chambers.  Immediately remove wet wrappers.

# Part 2 PRODUCTS

## MANUFACTURERS

- Acceptable Manufacturers:

- Products specified as standard of quality are manufactured by:

National Access Design, LLC

1924 Losantiville Ave.

Cincinnati, OH. 45237

1-513-351-3400

www.nationalaccessdesign.com

- Substitutions: Not permitted.

## MANUFACTURED UNITS

- Traffic Doors:

- Acceptable product: National Access Design Model 6000.

- Traffic type: Retail/Commercial, personnel and cart traffic.

- Sizes:	As indicated on drawings (door schedule).

- Panels:

- Constructed of ¼” thick impact resistant ABS panel, jamb edge of panel reinforced with triple layer of ¼” ABS and finished with custom rigid fiberglass pultrusion.

- Color; Architect to chose color from standard manufacturer’s

Colors.

5.	Hinges:

- Hinges are G.S. style with hardened rollers, hinge covers and *bottom pin system*.

- Or *10” jamb guard*. (If not used should be removed at final draft)

6.	Vision Area:

- 9” X 14” clear vision panels held securely in place by low profile high strength plastic sealed frames.

- Or 6” x 12” (If panels are 42”Bi-parting or smaller) (If not used should be removed at final draft)

- Or 14” x 16” (If not used should be removed at final draft)

- Or 22” x 22” (If not used should be removed at final draft)

- Or 10” X30” ADA

- Panel Protection: Impact Plate

- None

- Or 12” high Impact plate(If not used should be removed at final draft)

- Or 18” high Impact plate (If not used should be removed at final draft)

- Or 24” high Impact plate (If not used should be removed at final draft)

- Or 36” high Impact plate (If not used should be removed at final draft)

- Or 42” high Impact plate (If not used should be removed at final draft)

- Or 48” high Impact plate (If not used should be removed at final draft)

# Part 3 EXECUTION

## EXAMINATION

- Verification of conditions:

- Verify traffic door frames are in accord with reviewed shop drawings.

- Examine conditions and substrates under which products of this section are to be installed; submit written notification of unacceptable conditions or substrates.

- Submit copy of installer’s report to Architect of record within 72 hours of report receipt.

- Proceeding with construction activity of this section:

- Prior to correction of unacceptable conditions or substrates are prohibited.

- Indicated installer’s acceptance of condition of product and substrates.

## INSTALLATION

- Steel tube frames: Or minimum double 2 X 4 wood, C-Channel or reinforced hollow metal frames. Installed in Metal Fabrication section.

- Traffic doors:

- Install in prepared openings in accordance with reviewed shop drawings and manufacturer’s

Installation instructions

- Plumb and level units.

- Adjust for proper operation.

- Clean surfaces per manufacturer’s instructions just prior to Date of Substantial Completion.

++ END OF SECTION ++